TRANA-TRACE

GPS-Based Covert Tracking & Emergency Alert System

Technologies: FPGA  ·  NodeMCU ESP8266  ·  Node.js  ·  React  ·  OpenCV  ·  Android CameraX  ·

Firebase ML Kit

# Abstract

Trana-Trace is a covert GPS tracking and emergency alert system designed for personal safety. It combines FPGA-based deterministic state control, IoT communication via NodeMCU ESP8266, AI-powered face recognition on a companion Android device, and a real-time React dashboard. The key innovation is Fake-Off Mode — the device continues covert tracking even when it appears powered off. An unauthorized shutdown attempt triggers automatic intruder detection, forensic evidence capture, and an instant Telegram alert to the owner.

# Team Naviel

| Name | Roll Number | Primary Contribution |
| --- | --- | --- |
| G. Likhita | 24B11EC099 | FPGA Verilog Design, FSM Implementation |
| K. Tanuja | 24B11EC146 | FSM Implementation and verification |
| P. Sowmya | 24B11EC253 | Backend (Node.js, Redis, WebSocket) |
| U. Abhilash Joyeal | 24B11EC324 | IOT Developer |
| G. Guru Nandhini | 24B11EC085 | Embedded Developer |
| B. Dheeraja | 24B11EC024 | Frontend |

# 1. Overview

Trana-Trace is a real-time GPS tracking and emergency alert system designed for personal safety applications. The system integrates FPGA-based hardware control, GPS tracking, NodeMCU IoT communication, computer-vision-based face recognition, and a live web monitoring dashboard into a unified covert tracking and emergency alert platform.

Main Features:

- Live GPS tracking with real-time dashboard

- Emergency alert transmission via Telegram

- Stealth (Fake-Off) mode — device appears OFF while tracking continues

- Smart Authorized Shutdown using face recognition

- Unauthorized intruder detection and evidence capture

- WebSocket-based real-time dashboard updates

A key innovation is the Fake-Off Mode — the device appears powered OFF while secretly continuing tracking in the background, reinforced by Smart Authorized Shutdown that uses face recognition to decide between a real or fake shutdown.

# 2. Problem Statement

Traditional tracking systems stop functioning when a device is powered OFF. In personal safety scenarios — theft, abduction, or forced shutdown — a perpetrator can simply power off the device to eliminate tracking. Trana-Trace implements a covert tracking mechanism that continues background monitoring even in stealth mode, combined with AI face recognition to decide whether a shutdown is authorized.

# 3. Comparison with Existing Systems

| Feature | Apple AirTag | Samsung SmartTag | Trana-Trace |
| --- | --- | --- | --- |
| Continues tracking when OFF | No | No | Yes (Fake-Off Mode) |
| Face-based shutdown auth | No | No | Yes |
| Intruder evidence capture | No | No | Yes (image + GPS) |
| Telegram emergency alert | No | No | Yes |
| FPGA deterministic control | No | No | Yes |
| Custom web dashboard | No | No | Yes |
| Open / hackable hardware | No | No | Yes |

# 4. Objectives

- Develop a covert GPS tracking system that operates even in apparent OFF state

- Implement FPGA-based deterministic state control

- Provide real-time monitoring using a web dashboard

- Send emergency alerts instantly via Telegram with image and location

- Integrate face recognition for authorized shutdown verification

- Combine FPGA, IoT, and AI technologies into a single safety platform

# 5. System Architecture Overview

## Layer 1 — Hardware Control (FPGA + NodeMCU)

The FPGA manages the Finite State Machine and processes push-button input with hardware-level debounce. State changes are communicated to the NodeMCU via UART. The NodeMCU reads GPS coordinates, connects to WiFi, communicates with the backend server, and manages the OLED display.

## Layer 2 — Mobile Application (Android)

A companion Android application handles computer-vision operations too resource-intensive for the NodeMCU. When a shutdown is triggered, the app activates the front camera, runs face recognition using

Firebase ML Kit and OpenCV, and sends the result (plus an intruder image if unrecognized) to the backend.

## Layer 3 — Backend & Dashboard (Node.js + React)

A Node.js backend with Redis stores live device data and broadcasts real-time updates via WebSocket. The

React dashboard displays GPS location on a Leaflet map, shows device status, and allows manual SOS triggering.

# 6. Working Principle

## Normal Operation Flow

Button Press → FPGA FSM → UART → NodeMCU → GPS Read (NEO-6M) → Backend API → Redis → WebSocket → React Dashboard + Telegram Alert

## Shutdown Authorization Flow

OFF Button → FPGA signals shutdown → NodeMCU notifies Android via backend → Camera activates → Face recognition → Authorized? REAL_OFF : FAKE_OFF → If FAKE_OFF: image + GPS + timestamp captured → Telegram alert → Covert tracking continues

# 7. Hardware Architecture

| Component | Function |
| --- | --- |
| FPGA (Tang Nano / equivalent) | Controls FSM, click detection, hardware debounce |
| NodeMCU ESP8266 | WiFi communication, backend connectivity, GPS reading |
| NEO-6M GPS Module | Provides real-time GPS coordinates |
| OLED Display (128×64) | Displays current device status |
| Push Button | User input for state transitions |
| LEDs | Indicate operating state visually |

# 8. Technical Specifications

| Parameter | Value |
| --- | --- |
| FPGA Clock Frequency | 27 MHz |
| UART Baud Rate | 9600 bps |
| Debounce Delay | 20 ms |
| Click Detection Window | 1 second |
| GPS Module | NEO-6M |
| Display Resolution | 128 × 64 pixels |
| Communication Protocol | UART (FPGA ↔ NodeMCU) |
| Database | Redis (localhost:6379) |
| Face Recognition Engine | Firebase ML Kit + OpenCV |
| Mobile Platform | Android (CameraX API) |

# 9. FPGA State Machine

## 9.1 State Definitions

| State | Code | Behavior |
| --- | --- | --- |
| NORMAL | 0 | Live GPS coordinates displayed on OLED; tracking active |
| FAKE_OFF | 1 | Display OFF; covert tracking continues in background |
| EMERGENCY | 2 | SOS mode with rapid LED blinking; Telegram alert sent |
| REAL_OFF | 3 | Complete system shutdown after authorized face verification |

## 9.2 State Transition Table

| From State | Trigger / Input | Condition | To State |
| --- | --- | --- | --- |
| NORMAL | Single click | Always | FAKE_OFF |
| NORMAL | Hold button (3 s) | Always | EMERGENCY |
| NORMAL | Double click | Always | EMERGENCY |
| FAKE_OFF | Shutdown request | Face matches owner | REAL_OFF |
| FAKE_OFF | Shutdown request | Face does not match | FAKE_OFF (stays) |
| EMERGENCY | Single click | Always | NORMAL |
| EMERGENCY | Telegram SOS ack | Backend confirmation | NORMAL |
| REAL_OFF | — | System powered off | — |

Hardware-level debounce (20 ms) prevents false triggering. FPGA determinism guarantees transitions occur within a fixed number of clock cycles with no OS jitter.

# 10. Firmware (gpsbot.ino)

NodeMCU firmware operations:

- Reads GPS data using TinyGPSPlus library

- Uses fallback static coordinates when GPS lock is unavailable

- Reads FPGA state through UART

- Sends Telegram alerts with location and status

- Sends JSON device data to backend server

- Updates OLED display based on current system state

- Notifies Android companion app via backend API on shutdown request

Libraries Used:

- TinyGPSPlus — GPS data parsing

- Adafruit_SSD1306 — OLED display control

- UniversalTelegramBot — Telegram messaging

- ESP8266HTTPClient — HTTP backend communication

- SoftwareSerial — Secondary UART for FPGA communication

# 11. Backend (Node.js)

Technologies: Node.js + Express, Redis (live state), WebSocket (real-time broadcast), REST APIs.

| Endpoint | Method | Purpose |
| --- | --- | --- |
| /update | POST | Receives device GPS and state data from NodeMCU |
| /device | GET | Returns current device state to dashboard |
| /api/sos | POST | Manual SOS trigger from dashboard |
| /api/shutdown-request | POST | Receives shutdown notification; triggers Android face check |
| /api/intruder-report | POST | Receives intruder image, GPS, timestamp from Android |

# 12. Frontend (React Dashboard)

Technologies: React + Vite, Tailwind CSS, Framer Motion, Leaflet.

Dashboard Features:

- Live GPS tracking map with real-time coordinate updates

- Device status display (ACTIVE / OFFLINE / DANGER / FAKE_OFF)

- Real-time activity feed

- Manual SOS trigger button

- Automatic WebSocket reconnection on disconnect

- Responsive interface for mobile and desktop

# 13. Smart Authorized Shutdown System

13.1 Overview

This is a core safety feature that determines whether a shutdown request is legitimate or unauthorized.

## 13.2 Face Registration Process

Before deploying the device, the owner must register their face in the companion Android app:

- Open the Android app and navigate to Settings → Register Owner Face.

- The front camera captures 5–10 reference frames under varying lighting conditions.

- Firebase ML Kit processes and stores the face embedding locally on the device.

- Only one owner profile is supported in the current version (multi-user support is planned).

- Face data is stored in local secure storage and is never uploaded to any cloud service.

## 13.3 Shutdown Authorization Workflow

- User presses OFF button; FPGA detects and signals shutdown request.

- NodeMCU notifies the companion Android app via backend API.

- Android app activates the front camera automatically.

- Face recognition verifies the user against the stored owner profile.

- If authorized: REAL_OFF state — complete system shutdown.

- If unauthorized: FAKE_OFF state — tracking continues silently; intruder image, GPS, and timestampsent via Telegram.

| Outcome | Action |
| --- | --- |
| Face matches registered owner | REAL_OFF — complete shutdown executed normally |
| Face does not match / unrecognized | FAKE_OFF — display powers off; covert tracking continues silently |

# 14. Unauthorized Intruder Detection & Evidence Capture

When an unauthorized shutdown attempt is detected, the system silently collects forensic evidence:

- Intruder image captured automatically using front camera

- Current GPS coordinates fetched from NEO-6M module

- Timestamp recorded at the moment of detection

- Captured data uploaded to backend server

- Telegram alert sent to owner with image and GPS location

- Covert tracking maintained in the background (FAKE_OFF state)

# 15. Complete Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Hardware FSM | FPGA (Verilog) | Deterministic state control, click detection, debounce |
| Embedded / IoT | NodeMCU ESP8266 | WiFi, UART, GPS reading, backend communication |
| GPS | NEO-6M + TinyGPSPlus | Real-time coordinate acquisition |
| Mobile (AI) | Android + CameraX | Camera access for face verification |
| Face Recognition | Firebase ML Kit + OpenCV | Owner verification on shutdown attempt |
| Backend | Node.js + Express | API, state logic, data relay |
| Database | Redis | Live device state storage |
| Real-time Comms | WebSocket | Push updates from backend to dashboard |
| Alerts | Telegram Bot API | Instant alerts with image and GPS |
| Frontend | React + Vite + Leaflet | Live monitoring dashboard |
| Storage (Evidence) | Cloud Storage | Intruder images and logs |

# 16. Setup and Execution

## Backend Setup

- Open terminal in the backend folder.

- Run: npm install

- Ensure Redis is running on localhost:6379.

- Run: node server.js

## Frontend Setup

- Open terminal in the frontend folder.

- Run: npm install

- Run: npm run dev

## Firmware Setup

- Create config.h with WiFi credentials, Telegram bot token, chat ID, and backend IP.

- Add config.h to .gitignore — never upload credentials.

- Flash firmware to NodeMCU using Arduino IDE.

- Upload Verilog files to FPGA using appropriate toolchain.

## Android App Setup

- Open the Android project in Android Studio.

- Configure Firebase ML Kit credentials.

- Register the owner’s face via Settings → Register Owner Face (5–10 frames required).

- Set the backend server IP and install the APK on the companion device.

# 17. Testing & Results

| Test Scenario | Result | Notes |
| --- | --- | --- |
| GPS lock — outdoor | Pass | Lock acquired within ~45 s; accuracy ~3 m |
| GPS lock — indoor | Partial | Falls back to static coordinates; A-GPS planned |
| UART sync (FPGA ↔ NodeMCU) | Pass | Baud rate 9600 stable; occasional re-sync on power cycle |
| Fake-Off covert tracking | Pass | Display off; WebSocket updates continued without interruption |
| Emergency alert (Telegram) | Pass | Alert delivered within 2–4 s of EMERGENCY trigger |
| Authorized shutdown (face match) | Pass | REAL_OFF correctly triggered for registered owner face |
| Unauthorized shutdown (face mismatch) | Pass | FAKE_OFF triggered; intruder image + GPS sent via Telegram |
| WebSocket reconnection | Pass | Dashboard auto-reconnected within 3 s after network drop |

Known issue: GPS fallback to static coordinates in indoor environments is a hardware limitation of the NEO-6M module.

Targeted for improvement using A-GPS in the next iteration.

# 18. Technical Importance of FPGA

Using an FPGA for state control — rather than a microcontroller — provides three key advantages:

- Deterministic timing: State transitions happen in a fixed, guaranteed number of clock cycles with no OS-induced jitter.

- Hardware debounce: Push-button noise is filtered at the hardware level (20 ms window), eliminating false triggers that software debounce can miss under interrupt load.

- Parallel logic: Click pattern detection and UART transmission can operate concurrently without software scheduling overhead.

These properties are especially important in emergency situations where even millisecond delays in state transitions could matter.

# 19. Security Notes

| Area | Current Status | Recommended Improvement |
| --- | --- | --- |
| API Authentication | Not implemented | Add JWT-based authentication |
| WebSocket | ws:// (unencrypted) | Upgrade to wss:// with TLS |
| API Transport | HTTP | Implement HTTPS with SSL certificate |
| Credentials | Stored in config.h | Use environment variables (.env) in production |
| Intruder Images | Uploaded to backend | Encrypt at rest; restrict access with auth |
| GitHub | config.h excluded via .gitignore | Verify .gitignore is committed before first push |

# 20. Known Limitations

| Limitation | Impact | Planned Fix |
| --- | --- | --- |
| Battery % fixed at 100% | No real power monitoring | Add INA219 battery monitoring hardware |
| GPS falls back indoors | Inaccurate coordinates indoors | Integrate A-GPS or Wi-Fi geolocation API |
| No API authentication | Backend open to unauthorized calls | Implement JWT authentication |
| WebSocket unencrypted (ws://) | Data exposed on network | Upgrade to wss:// with TLS |
| Duplicate /update endpoints | Potential race conditions | Consolidate to single endpoint |
| Face recognition needs Android | Separate device required | Explore TensorFlow Lite on future hardware |

# 21. Approximate Hardware Cost

| Component | Estimated Cost (INR) | Notes |
| --- | --- | --- |
| FPGA Board (Tang Nano 9K equivalent) | 1,200 – 2,500 | Sipeed Tang Nano or similar |
| NodeMCU ESP8266 | 150 – 300 | Widely available |
| NEO-6M GPS Module | 300 – 500 | Includes antenna |
| OLED Display (128×64) | 100 – 200 | I2C interface |
| Push Buttons, LEDs, Misc | 50 – 100 | Resistors, wires, PCB |
| Android Device (companion) | Existing device | Uses owner’s smartphone |
| Total (estimated) | 1,800 – 3,600 | Excluding Android device |

# 22. Real-World Applications

- Women’s safety devices — covert tracking continues even if phone appears off

- Anti-theft tracking — vehicle or asset continues reporting despite apparent shutdown

- Child safety applications — parents receive alerts if device is tampered with

- Emergency response systems — instant SOS with GPS and photo evidence

- Smart security devices — unauthorized access triggers automatic evidence capture

# 23. Advantages of the System

| Advantage | Description |
| --- | --- |
| Covert tracking | Tracking continues invisibly even when device appears powered off |
| AI-based authorization | Face recognition ensures only the owner can perform a real shutdown |
| Forensic evidence | Intruder images, GPS, and timestamps automatically captured and sent |
| Deterministic FPGA control | Hardware FSM guarantees response time with no software jitter |
| Real-time dashboard | Live GPS and status updates over WebSocket without page refresh |
| Multi-channel alerts | Telegram delivers image + location instantly to the owner |

# 24. Future Improvements

- Battery monitoring hardware (INA219) to replace fixed 100% value

- Encrypted communication (HTTPS + WSS) for production deployment

- A-GPS integration using Google Geolocation API for improved indoor accuracy

- On-device edge ML (TensorFlow Lite) to eliminate dependency on companion Android app

- JWT-based user authentication for backend and dashboard

- Cloud database migration (Firebase Firestore) for persistent logging

- Multi-user face registration support for shared device ownership

- Dedicated iOS companion app

# 25. Challenges Faced

- GPS signal instability in indoor environments — mitigated using static coordinate fallback

- UART synchronization between FPGA and NodeMCU — resolved by standardizing baud rate andadding re-sync on power cycle

- Real-time WebSocket communication handling — resolved with auto-reconnect logic in the Reactdashboard

- Power optimization during stealth mode — minimized NodeMCU active duty cycle to reduce batterydrain

- Coordinating FPGA state machine with Android app — required careful API design to ensure shutdowntriggers reach the mobile app reliably

# 26. Conclusion

Trana-Trace is an intelligent, safety-oriented tracking system that successfully integrates FPGA hardware control, embedded IoT firmware, computer-vision-based face recognition, and a real-time web dashboard into a single cohesive platform.

The system demonstrates:

- Covert tracking that survives apparent device shutdown

- AI-powered authorization using face recognition on a companion Android device

- Real-time forensic evidence collection during unauthorized access attempts

- Deterministic FPGA state control for reliable emergency response

- End-to-end integration of FPGA, IoT, AI, and web technologies

The project establishes a strong foundation for next-generation personal safety devices. With planned improvements in encrypted communication, on-device ML, and cloud integration, Trana-Trace has clear potential for real-world deployment in safety-critical applications.

# Glossary

| Term | Definition |
| --- | --- |
| FPGA | Field-Programmable Gate Array — reconfigurable hardware chip for deterministic logic |
| FSM | Finite State Machine — model with a fixed set of states and transitions |
| UART | Universal Asynchronous Receiver-Transmitter — serial communication protocol |
| GPS | Global Positioning System — satellite-based location tracking |
| A-GPS | Assisted GPS — uses network data to speed up satellite lock indoors |
| CameraX | Android Jetpack library for camera access with simplified API |
| Firebase ML Kit | Google’s on-device machine learning SDK for Android |
| WebSocket | Full-duplex communication protocol over a single TCP connection |
| Redis | In-memory key-value database used for live state storage |
| JWT | JSON Web Token — compact token format for secure authentication |
| TLS/WSS | Transport Layer Security / WebSocket Secure — encrypted communication protocols |
| NEO-6M | u-blox GPS module commonly used in hobbyist and embedded applications |

TRANA-TRACE — Built at Aditya University | Team Naviel